ANEXO III – VALORES ORIENTADORES PARA SOLO E ÁGUA SUBTERRÂNEA

| Grupo | Substâncias | CAS n° | Referência de Qualidade | Prevenção | Investigação |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Solo (mg.Kg-1) |  |  | Água Subterrânea (µg.L-1) | Ecológico (mg.Kg-1) |
|  |  |  | Solo (mg.Kg-1) |  | Agrícola | Residencial | Industrial |  |  |
| Inorgânicos * | Alumínio | 7429-90-5 | E | - | - | - | - | 3500 |  |
| Inorgânicos * | Antimônio | 7440-36-0 | E | 2 | 5 | 10 | 25 | 6 (1) | 374 (2) |
|  | Arsênio | 7440-38-2 | E | 15 | 35 | 55 | 150 | 10(1) | 0,02 (4) |
|  | Bário | 7440-39-3 | E | 120 (15) | Ibama: 300 Abema: 500(15) | Ibama: 500 Abema: 1300(15) | Ibama: 750 Abema: 7300(15) | 700 (1) | Ibama: 41,50 (2) Abema: 178 |
|  | Boro | 7440-42-8 | E | - | - | - | - | 2400 (15) | Ibama: 2,10 (2) Abema: exclusão |
|  | Cádmio | 7440-43-9 | E | 1,3 | Ibama: 3 Abema: 3,6 (15) | Ibama: 8 Abema: 14 (15) | Ibama: 20 Abema: 160 (15) | 3 (1) | 3,1 (8) |
|  | Chumbo | 7439-92-1 | E | 72 | 200 (6) | 200 (6) | 800 (6) | 10 (1) | 167 (8) |
|  | Cianeto | 57-12-5 | E | - | 0,9 (3) | 0,9 (3) | 8 (3) | 50 (9) | 0,1 Abema: exclusão |
|  | Cobalto | 7440-48-4 | E | 25 | 35 | 65 | 90 | 70 | 1,86 (7) |
|  | Cobre | 7440-50-8 | E | 60 | Ibama: 200 Abema: 760(15) | Ibama: 400 Abema: 2100(15) | Ibama: 600 Abema: 10000(15) | 2.000 (1) | 15,5 (8) |
|  | Cromo total | 7440-47-3 | E | 75 | 150 | 300 | 400 | 50 (1) | Ibama: 6,75 (8) Abema: 69 |
|  | Cromo VI | 18540-29-9 | E | - | 0,4 (3) | 3,2 (15) | 10 (15) | 10 (10) | 0,4 (11) |
|  | Estanho | 7440-31-5 | E | - | 5 (3) | 50 (3) | 300 (3) | 50 (12) | Ibama: 4,12 (2) Abema: exclusão |
|  | Ferro | 7439-89-6 | E | - | - | - | - | 2400(1) | - |
|  | Manganês | 7439-96-5 | E | - | - | - | - | 400(1) | 220 (14) |
|  | Mercúrio | 7439-97-6 | E | 0,5 | 1,2 (15) | 0,9 (15) | 7 (15) | 1 (1) | 4,2 (8) |
|  | Metilmercúrio | 22967-92-6 | E | - | - | - | - | - | Ibama: 0,37 (8) Abema: exclusão |
|  | Molibdênio | 7439-98-7 | E | 5 (15) | 11 (3) | 29 (3) | Ibama: 120 Abema: 180 (15) | 30 (15) | 14,30 (7) |
|  | Níquel | 7440-02-0 | E | 30 | Ibama: 70 Abema: 190 | Ibama: 100 Abema: 480 | Ibama: 130 Abema: 3800 | 70 (1) | 37,4 (16) |
|  | Nitrato (como N) | 14797-55-8 | E | - | - | - | - | 10.000 (1) | - |
|  | Prata | 7440-22-4 | E | 2 | 25 | 50 | 100 | 50 | 4,2 (14) |
|  | Selênio | 7782-49-2 | E | 1,2 (15) | 24 (15) | 81 (15) | 640 (15) | 40 (1) | 0,083 (2) |
|  | Urânio(m) | 7440-61-1 | E | - | 23 (3) | 23 (3) | 300 (3) | 30 (1) | - |
|  | Vanádio | 7440-62-2 | E | - | - | - | - | 1000 | Ibama: 0,89 (2) Abema: exclusão |
|  | Zinco | 7440-66-6 | E | 86 (15) | Ibama: 450 Abema: 1900(15) | Ibama: 1000 Abema: 7000(15) | Ibama: 2000 Abema: 10000(15) | Ibama: 1.050 Abema: 1800 (15) | 58 (8) |
| Hidrocarbonetos aromáticos voláteis (HAV) | Benzeno | 71-43-2 | na | 0,002 (15) | 0,02 (15) | 0,08 | 0,15 | 5 (1) | 0,08 (8) |
|  | Estireno | 100-42-5 | na | 0,2 | Ibama: 15 Abema: 50 (15) | Ibama: 35 Abema: 60 (15) | Ibama: 80 Abema: 480 (15) | 20 | 1,9 (8) |
|  | Etilbenzeno | 100-41-4 | na | 0,03 (15) | 0,2 (15) | 0,6 (15) | 1,4 (15) | 300 (1) | 26 (8) |
|  | Tolueno | 108-88-3 | na | 0,14 | 5,6 (15) | 14 (15) | 80 (15) | 30 (1) | 10,95 (8) |
|  | Xilenos | 1330-20-7 | na | 0,03 (15) | 12 (15) | 3,2 (15) | 19 (15) | 500 (1) | 1,49 (8) |
| Hidrocarbonetos policíclicos aromáticos (HPAs) | Antraceno | 120-12-7 | na | 0,3 (15) | 2.300 (15) | 4600 (15) | 10.000 (15) | 900 (15) | 4,5 (17) |
|  | Benzo(a)antraceno | 56-55-3 | na | 0,2 (15) | 1,6 (15) | 7 (15) | 22 (15) | 0,4 (15) | 4,2 (17) |
|  | Benzo(b)fluoranteno | 205-99-2 | na | 0,7 (15) | 2 (15) | 7,2 (15) | 25 (15) | 0,4 (15) | 7,00 (17) |
|  | Benzo(k)fluoranteno | 207-08-9 | na | 0,8 (15) | 27 (15) | 75 (15) | 240 (15) | 4,1 (15) | 5,90 (17) |
|  | Benzo(g,h,i)perileno | 191-24-2 | na | 0,5 (15) | - | - | - | - | 2,2 (17) |
|  | Benzo(a)pireno | 50-32-8 | na | 0,1 (15) | 0,2 (15) | 0,8 (15) | 2,7 (15) | 0,4 (1) | 3,5 (17) |
|  | Criseno | 218-01-9 | na | 1,6 (15) | 95 (15) | 600 (15) | 1.600 (15) | 41 (15) | 7,8 (17) |
|  | Dibenzo(a,h)antraceno | 53-70-3 | na | 0,2 (15) | Ibama: 0,15 Abema: 0,3 (15) | Ibama: 0,6 Abema: 0,8 (15) | Ibama: 1,3 Abema: 2,9 (15) | 0,04 (15) | 1,80 (17) |
|  | Fenantreno | 85-01-8 | na | 3,6 (15) | 15 | 40 | 95 | 140 | 18 (17) |
|  | Indeno(1,2,3-c,d)pireno | 193-39-5 | na | 0,4 (15) | Ibama: 2 Abema: 3,4 (15) | 8 (15) | 30 (15) | Ibama: 0,17 Abema: 0,4 (15) | 5,8 (17) |
|  | Naftaleno | 91-20-3 | na | 0,7 (15) | 1,1 (15) | 1,8 (15) | 5,9 (15) | 60 (15) | 3,11 (17) |
|  | Acenafteno | 83-32-9 | na | 0,68 (24) | 120 (24) | 340 (24) | 730 (24) | 18 (24) | 4,59 (17) |
|  | Fluoranteno | 206-44-0 | na | 0,01(24) | 520 (24) | 1300 (24) | 4800 (24) | 12 (24) | 38,51(17) |
|  | Fluoreno | 86-73-7 | na | 1,6 (24) | 130 (24) | 370 (24) | 870 (24) | 12 (24) | 11,45 (17) |
|  | Pireno | 129-00-0 | na | 1,8 (24) | 400 (24) | 940 (24) | 3500 (24) | 9 (24) | 9,77 (17) |
|  | 1-Metilnaftaleno | 90-12-0 | na | 0,09 (24) | 0,55 (24) | 3,9 (24) | 6,7 (24) | 10 (24) | - |
|  | 2-Metilnaftaleno | 91-57-6 | na | 0,07 (24) | 4 (24) | 12 (24) | 25 (24) | 1,2 (24) | - |
|  | 4-Nitropireno | 57835-92-4 | na | 0,07 (24) | 0,35 (24) | 2,4 (24) | 4,5 (24) | 0,25 (24) | - |
| Benzenos clorados | Clorobenzeno (Mono) | 108-90-7 | na | 0,3 (15) | 1,6 (15) | 1,3 (15) | 8,3 (15) | 120 (15) | 2,5 (8) |
|  | 1,2-Diclorobenzeno | 95-50-1 | na | 0,7 (15) | 9,2 (15) | 11 (15) | 84 (15) | 1.000 | 3,5 (8) |
|  | 1,3-Diclorobenzeno | 541-73-1 | na | 0,4 (15) | - | - | - | - | 3,1 (8) |
|  | 1,4-Diclorobenzeno | 106-46-7 | na | 0,1 (15) | 0,3 (15) | 0,6 (15) | 2,1 (15) | 300 | 2,4 (18) |
|  | 1,2,3-Triclorobenzeno | 87-61-6 | na | 0,01 | 0,4 (15) | 1,1 (15) | 6,1 (15) | (b) | 0,22 (8) |
|  | 1,2,4-Triclorobenzeno | 120-82-1 | na | 0,01 (15) | 0,4 (15) | 1 (15) | 8,4 (15) | (b) | 0,24 (8) |
|  | 1,3,5-Triclorobenzeno | 108-70-3 | na | 0,5 | - | - | - | (b) | 5,00 (8) |
|  | 1,2,3,4-Tetraclorobenzeno | 634-66-2 | na | 0,003 (15) | - | - | - | - | 0,2 (18) |
|  | 1,2,3,5-Tetraclorobenzeno | 634-90-2 | na | 0,006 (15) | - | - | - | - | 0,07 (8) |
|  | 1,2,4,5-Tetraclorobenzeno | 95-94-3 | na | 0,01 | 0,3 (15) | 0,6 (15) | 3,6 (15) | 1,8 (15) | 0,10 (8) |
|  | Hexaclorobenzeno | 118-74-1 | na | 0,02 (15) | 0,2 (15) | 1,3 (15) | 3,4 (15) | 0,2 (15) | 0,22 (8) |
| Etanos clorados | 1,1-Dicloroetano | 75-34-3 | na | 0,02 (15) | 0,1 (15) | 0,6 (15) | 1,7 (15) | 53 (15) | - |
|  | 1,2-Dicloroetano | 107-06-2 | na | 0,001 (15) | 0,01 (15) | 0,03 (15) | 0,09 (15) | 5 (1) | 23 (19) |
|  | 1,1,1-Tricloroetano | 71-55-6 | na | 0,2 (15) | Ibama: 11 Abema: 140 (15) | Ibama: 11 Abema: 120 (15) | Ibama: 25 Abema: 690 (15) | Ibama: 280 Abema: 2000 (15) | 1,3 (19) |
| Etenos clorados | Cloreto de vinila | 75-01-4 | na | 0,0002 (15) | 0,001 (15) | Ibama: 0,003 Abema: 0,01 (15) | Ibama: 0,008 Abema: 0,03 (15) | 0,5 (1) | 0,05 (20) |
|  | 1,1-Dicloroeteno | 75-35-4 | na | 0,04 (15) | 2,8 (15) | Ibama: 3 Abema: 3,8 | Ibama: 8 Abema: 22 | 30 | 1,5 (20) |
|  | 1,2-Dicloroeteno - cis | 156-59-2 | na | 0,01 (15) | 0,08 (15) | 0,2 (15) | 1,1 (15) | (c) | 0,77 (8) |
|  | 1,2-Dicloroeteno - trans | 156-60-5 | na | 0,03 (15) | 0,7 (15) | 1 (15) | 5,4 (15) | (c) | 1,11 (8) |
|  | Tricloroeteno - TCE | 79-01-6 | na | 0,004 (15) | 0,03 (15) | 0,04 (15) | 0,2 (15) | 4 (1) | 4,3 (19) |
|  | Tetracloroeteno - PCE | 127-18-4 | na | 0,03 (15) | 0,6 (15) | 0,8 (15) | 4,6 (15) | 40 (1) | 0,52 (19) |
| Metanos clorados | Cloreto de Metileno | 75-09-2 | na | 0,02 (15) | 0,1 (15) | 0,4 (15) | 2,1 (15) | 20 (1) | 25 (19) |
|  | Clorofórmio | 67-66-3 | na | 0,06 (15) | 0,1 (15) | 0,8 (15) | 4,5 (15) | Ibama: 200 Abema: 300 (15) | 6,6 (19) |
|  | Tetracloreto de carbono | 56-23-5 | na | 0,004 (15) | 0,03 (15) | 0,1 (15) | 0,4 (15) | 4 (1) | 1,4 (19) |
| Fenóis clorados | 2-Clorofenol (o) | 95-57-8 | na | 0,06 (15) | 0,6 (15) | 1,7 (15) | Ibama: 2 Abema: 9,4 (15) | Ibama: 10,5 Abema: 30 (15) | 0,65 (8) |
|  | 2,4-Diclorofenol | 120-83-2 | na | 0,03 (15) | 0,5 (15) | 1,5 (15) | 8,5 (15) | 200 (1) | 0,51 (8) |
|  | 3,4-Diclorofenol | 95-77-2 | na | 0,05 | 1 | 3 | 6 | 10,5 | 1,2 (8) |
|  | 2,4,5-Triclorofenol | 95-95-4 | na | 0,1 (15) | 68 (15) | 170 (15) | 960 (15) | 10,5 | 1,6 (8) |
|  | 2,4,6-Triclorofenol | 88-06-2 | na | 0,1 (15) | 0,6 (15) | 1,6 (15) | 9,6 (15) | 200 (1) | 4,4 (8) |
|  | 2,3,4,5-Tetraclorofenol | 4901-51-3 | na | 0,09 | 7 | 25 | 50 | 10,5 | 2,4 (8) |
|  | 2,3,4,6-Tetraclorofenol | 58-90-2 | na | 0,01 | Ibama: 1 Abema: 34 (15) | Ibama: 3,5 Abema: 85 (15) | Ibama: 7,5 Abema: 480 (15) | Ibama: 10,5 Abema: 180 (15) | 0,38 (8) |
|  | Pentaclorofenol (PCP) | 87-86-5 | na | 0,01 (15) | 0,07 (15) | 0,6 (15) | 1,9 (15) | 9 (1) | 1,4 (8) |
| Fenóis não clorados | Cresóis totais | 1319-77-3 | na | 0,16 | Ibama: 6 Abema: 14 (15) | Ibama: 14 Abema: 33 (15) | Ibama: 19 Abema: 190 (15) | Ibama: 175 Abema: 600 (15) | 1,4 (8) |
|  | Cresol-m | 108-39-4 | na | 0,2 (24) | 8,4 (24) | 22 (24) | 43 (24) | 300 (24) | 1,6 (8) |
|  | Cresol-o | 95-48-7 | na | 0,5 (8) | 8,3 (24) | 22 (24) | 43 (24) | 300 (24) | 5 (8) |
|  | Cresol-p | 106-44-5 | na | 0,005 (15) | - | - | - | - | 0,12 (8) |
|  | Fenol | 108-95-2 | na | 0,20 | Ibama: 5 Abema: 24 (15) | Ibama: 10 Abema: 65 (15) | Ibama: 15 Abema: 370 (15) | Ibama: 140 Abema: 900 (15) | 1,7 (8) |
| Ésteres ftálicos | Dietil ftalato | 84-66-2 | na | 0,5 (15) | 33 (15) | 100 (15) | 550 (15) | 4,8 (15) | 5,30 (8) |
|  | Dietilexil ftalato (DEHP) | 117-81-7 | na | 1 (15) | Ibama: 1,2 Abema: 36 (15) | Ibama: 4 Abema: 250 (15) | Ibama: 10 Abema: 730 (15) | 8 (1) | 8,3 (8) |
|  | Dimetil ftalato (DMP) | 131-11-3 | na | 0,25 | 0,5 | 1,6 | 3 | 14 | Ibama: 9,17 (8) Abema: 4,6 |
|  | Di-n-butil ftalato (DBP) | 84-74-2 | na | 0,1 (15) | 44 (15) | 140 (15) | 850 (15) | 600 (15) | 4,6 (8) |
| Agrotóxicos | Atrazina + S-Clorotriazinas (Deetil-Atrazina - Dea, Deisopropil-Atrazina - Dia e Diaminoclorotriazina -Dact) | 11912-24-9 (Atrazina) 6190-65-4 (Deetil-Atrazina - Dea) 1007-28-9 (Deisopropil-Atrazina - Dia) 3397-62-4 (Diaminoclorotriazina Dact) | na | Ibama / MMA / SC: inclusão. Abema: não inclusão | - | 2,4 (6) | 10 (6) | 2 (g) | 0,06 (8) |
|  | Bifentrina | 82657-04-3 | na | Ibama / MMA / SC: inclusão. Abema: não inclusão | 16 (15) | 1,7 (15) | 270 (15) | 120 | - |
|  | Carbofurano | 1563-66-2 | na | 0,0001 (15) | 0,3 (15) | 0,7 (15) | 3,8 (15) | 7 (1) | 0,001 (8) |
|  | Clorpirifós | 2921-88-2 | na | Ibama / MMA / SC: inclusão. Abema: não inclusão Abema: não inclusão | 16 (15) | 1,7 (15) | 270 (15) | 30 (h) | - |
|  | Metolacloro | 51218-45-2 | na | - | - | - | - | 10 (1) | - |
| Agrotóxicos Organoclorados | Aldrin | 309-00-2 | na | 0,015 | 0,039(n)(6) | 0,039(6) | 0,18(6) | (d) | 0,07 |
|  | Clordano | 57-74-9 | na | - | - | 1,7 (6) | 7,7 (6) | - | - |
|  | Dieldrin | 60-57-1 | na | 0,01 (15) | 0,3 (15) | 0,8 (15) | 1,3 | (d) | 0,10 (f) (21) |
|  | Endosulfam | 115-29-7 | na | 0,7 (15) | 4,7 (15) | 12 (15) | 66 (15) | 20 (15) | - |
|  | Endrin | 72-20-8 | na | 0,001 | Ibama: 0,4 Abema: 0,8 (15) | Ibama: 1,5 Abema: 2,5 (15) | Ibama: 2,5 Abema: 17 (15) | 0,6 | 0,01 (21) |
|  | DDT | 50-29-3 | na | 0,010 | Ibama: 0,55 Abema: 5,5 (15) | Ibama: 2 Abema: 22 (15) | Ibama: 5 Abema: 82 (15) | (e) (15) | 0,1 (8) |
|  | DDD | 72-54-8 | na | 0,02 (15) | 1 | Ibama: 3 Abema: 7,5 (15) | Ibama: 7 Abema: 23 (15) | (e) (15) | 0,78 (8) |
|  | DDE | 72-55-9 | na | 0,01 (15) | Ibama: 0,3 Abema: 1,2 (15) | Ibama: 1 Abema: 8,5 (15) | Ibama: 3 Abema: 25 (15) | (e) (15) | 0,12 (8) |
|  | HCH alfa | 319-84-6 | na | 0,0003 (15) | 0,002 (15) | 0,02 (15) | 0,04 (15) | 0,05 (15) | 2,30 (8) |
|  | HCH beta | 319-85-7 | na | 0,001 (15) | 0,01 (15) | 0,06 (15) | 0,2 (15) | Ibama: 0,07 Abema: 0,17 (15) | 0,38 (8) |
|  | HCH - gama (Lindano) | 58-89-9 | na | 0,001 | 0,008 (15) | 0,06 (15) | 0,2 (15) | 2 (1) | 0,03 (8) |
|  | Heptacloro | 76-44-8 | na | - | 0,13(n)(6) | 0,13 (6) | 0,63 (6) | 0,4 (5) | - |
|  | Mirex | 2385-85-5 | na | - | 0,036(n)(6) | 0,036 (6) | 0,17 (6) | 0,63 (a) (6) | - |
|  | Toxafeno | 8001-35-2 | na | - | 0,49(n)(6) | 0,49 (6) | 2,1 (6) | 3 (5) | - |
| Substâncias Perfluoralquiladas e Polifluoralquiladas (PFAS) | Ácido perfluoroctanóico (PFOA) e seus sais | 335-67-1 (PFOA) | na | Ibama / MMA / SC / Abema/ AESAS: inclusão. | 0,000019 (n)(6) | 0,000019 (6) | 0,000078 (6) | 0,004 (5) | - |
|  | Ácido perfluorobutanessulfônico (PFBS) e seus sais | 375-73-5 (PFBS) | na | Ibama / MMA / SC / Abema/ AESAS: inclusão. | 19 (n)(6) | 19 (6) | 250 (6) | 3 (a) (6) | - |
|  | Ácido perfluorodecanoico (PFUDA) e seus sais | 335-76-2 (PFUDA) | na | Ibama / MMA / SC / Abema/ AESAS: inclusão. | 19 (n)(6) | 19 (6) | 250 (6) | 6 (6) | - |
|  | Ácido perfluorohexano sulfônico (PFHxS) e seus sais | 355-46-4 (PFHxS) | na | Ibama / MMA / SC / Abema/ AESAS: inclusão. | 1,3 (n)(6) | 1,3 (6) | 16 (6) | 0,01 (5) | - |
|  | Ácido perfluorononanoico (PFNA) e seus sais | 375-95-1 (PFNA) | na | Ibama / MMA / SC / Abema/ AESAS: inclusão. | 0,19 (n)(6) | 0,19 (6) | 2,5 (6) | 0,01 (5) | - |
|  | Ácido perfluorooctano sulfônico (PFOS) e seus sais | 1763-23-1 (PFOS) | na | Ibama / MMA / SC / Abema/ AESAS: inclusão. 0,003 | 0,0063 (n)(6) | Abema:0.0063 (6) | Abema: 0,058 (6) | 0,004 (5) | 0,018 (22) |
|  | Óxido de hexafluoropropileno (HFPO-DA) e seu sal de amônio (GenX) | 13252-13-6 (HFPO-DA)/ 62037-80-3 (Gen-X) | na | Ibama / MMA / SC / Abema/ AESAS: inclusão. | 0,23 (n)(6) | 0,23 (6) | 3,5 (6) | 0,01 (5) | - |
| Hidrocarbonetos Totais de Petróleo (HTPs) | Alifáticos C19-C32 | (Código EPA E1790670) | na | - | 10.000(l) | 10.000(l) | 10.000(l) | 60.000 (k) (6) | - |
|  | Alifáticos C5-C8 | (Código EPA E1790666) | na | 12 | 15 | 250 (6) | 1900 (6) | 28 (6) | - |
|  | Alifáticos C9-C18 | (Código EPA E1790668) | na | 20 | 25 | 96 (6) | 440 (6) | 100 (6) | - |
|  | Aromáticos C10 -C32 | (Código EPA E1790676) | na | 15 | 18 | 18 (6) | 220 (6) | 6 (6) | - |
|  | Aromáticos C09-C10 | (Código EPA E1790674) | na | 12 | 15 | 300 (6) | 1700 (6) | 57 (6) | - |
| Outros | Anilina | 62-53-3 | na | 0,023 (15) | 0,15 (15) | 0,7 (15) | 3,2 (15) | 42 (15) | 0,95 |
|  | Dioxinas (PCDDs) e Furanos (PCDFs) (ng TEQ WHO05 Kg-1 peso seco) (j) | NA | na | 2 | 7,5 | 37 | 140 | - | 2 |
|  | PCBs Indicadores (i) |  | na | 0,0003 | 0,01 | 0,03 | 0,12 | 3,5 | 0,032 (8) |
|  | TBT e seus compostos (Óxido de Tributil) | NA | na | 0,24 (15) | 16 (15) | 1,7 (15) | 270 (15) | 0,09 (15) | 0,0055 (2) |

Notas: *Para as substâncias inorgânicas é necessário considerar os critérios descritos no artigo 47, parágrafos 1, 2, 3.

(a) - µg.Kg-1

(b) - somatório para triclorobenzenos = 20 µg.L-1

(c) - somatório para 1,2 dicloroetenos = 50 µg.L-1

(d) - somatório para Aldrin e Dieldrin = 0,03 µg.L-1

(e) - somatório para DDT-DDD-DDE = 1 µg.L-1

(f) - somatório para Aldrin e Dieldrin = 0,10 mg.kg-1

(g) - Atrazina + S-Clorotriazinas (Deetil-Atrazina - Dea, Deisopropil-Atrazina - Dia e Diaminoclorotriazina -Dact).

(h) - Clorpirifós + clorpirifós-oxon

(i) - somatório dos congêneres 28, 52, 101, 118,138,153,180 para investigação confirmatória; na investigação detalhada a lista de congêneres deve ser ampliada.

(j) - somatório de toxicidade equivalente (TEQ) calculada a partir dos fatores de equivalência de toxicidade (TEFs -WHO 2005) para cada congênere de dioxinas e furanos (VAN DEN BERG, 2006).

(k) – superior a solubilidade

(l) – adotado o valor limite de 1% peso seco do solo por exceder a concentração de saturação do solo.

(m) – Valor derivado exclusivamente com base nos efeitos toxicológicos, não abrangendo o gerenciamento ou a avaliação dos efeitos decorrentes da radioatividade o que será feito por órgão especializado.

(n) – Adotado o valor do RSL para o cenário residencial, por ser protetivo, em razão da importância da SQI para o cenário agrícola.

E - a ser definido pelo Estado

na - não se aplica para substâncias orgânicas

Referências: (1) BRASIL. Ministério da Saúde do Brasil. Portaria GM/MS nº 888, de 4 de maio de 2021. Altera o Anexo XX da Portaria de Consolidação GM/MS nº 5, de 28 de setembro de 2017, para dispor sobre os procedimentos de controle e de vigilância da qualidade da água para consumo humano e seu padrão de potabilidade. Disponível em: < https://www.in.gov.br/en/web/dou/-/portaria-gm/ms-n-888-de-4-de-maio-de-2021-318461562 >. Acesso em 10 abr. 2026.

(2) – HOLANDA. National Institute for Public Health and the Environment. Ministry of Health, Welfare and Sport. Proposal for Intervention Values soil and groundwater for de 2nd 3rd ant 4th series of compounds - RIVM report 607711006/2012 (2012). Disponível em: < https://www.rivm.nl/bibliotheek/rapporten/607711006.pdf >. Acesso em: 10 abr. 2026.

(3) - CANADÁ. CCME (Canadian Council of Ministers of the Environment) – Canadian Environmental Quality Guidelines — Summary Tables, Update 2002. Disponível: < https://ccme.ca/en/summary-table >. Acesso em: 10 abr. 2026.

(4) - HOLANDA. National Institute for Public Health and the Environment. Ministry of Health, Welfare and Sport. Update of ecological risk limits for arsenic in soil. RIVM Letter

report 2015-0138 (2015). Disponível em: < https://rivm.openrepository.com/server/api/core/bitstreams/10b2197a-df6f-4d9f-a93b-17b8946b5c47/content >. Acesso em: 10 abr. 2026.

(5) - E.U.A. United States Environmental Protection Agency (U.S.EPA) – National Primary Drinking Water Regulations. Disponível em: < https://www.epa.gov/ground-water-and-drinking-water/national-primary-drinking-water-regulations >. Acesso em: 10 abr. 2026.

(6) - E.U.A. United States Environmental Protection Agency (U.S.EPA) – Regional Screening Levels (RSLs) – Generic Tables – 2023. Disponível em: < https://www.epa.gov/risk/regional-screening-levels-rsls-generic-tables >. Acesso em: 10 abr. 2026.

(7) - HOLANDA. National Institute for Public Health and the Environment. Ministry of Health, Welfare and Sport. Environmental Risk Limits for Nine Trace Elements. RIVM report 601501029/2005 (2005). Disponível em: < https://www.rivm.nl/bibliotheek/rapporten/601501029.pdf >. Acesso em: 10 abr. 2026.

(8) - HOLANDA. National Institute for Public Health and the Environment. Ministry of Health, Welfare and Sport. Ecotoxicological Serious Risk Concentrations for soil, sediment and (ground)water: updated proposals for first series of compounds. Disponível em:< https://files01.core.ac.uk/download/pdf/58766155.pdf >. Acesso em: 10 abr. 2026.

(9) - REINO UNIDO. The Water Supply (Water Quality) Regulations, 2016 - SCHEDULE 1 – Prescribed concentrations and values. Disponível em: < https://www.legislation.gov.uk/uksi/2016/614/contents >. Acesso em: 10 abr. 2026.

(10) - E.U.A. CALIFÓRNIA. California State Water Resources Control Board. Hexavalent chromium (Chromium-6). Sacramento, CA, 2024. Disponível em: <https://www.waterboards.ca.gov/drinking_water/certlic/drinkingwater/Chromium6.html>. Acesso em: 10 abr. 2026.

(11) - CANADÁ. Environment Canada. Canadian soil quality guidelines for chromium: environmental supporting document – final draft. Ottawa: Guidelines Division, Science Policy and Environmental Quality Branch, 1996. Disponível em: <En108-4-80-1996-eng.pdf>. Acesso em: 10 abr. 2026.

(12) - HOLANDA. Netherlands Government – Soil remediation circular, 2013 – Anexo 1 – Groundwater target values and soil remediation intervention values. Disponível: < https://wetten.overheid.nl/BWBR0033592/2013-07-01 >. Acesso em: 10 abr. 2026.

(13) - AUSTRÁLIA. Australian National Environment Protection 1999.

(14) - E.U.A. United States Environmental Protection Agency (U.S.EPA). Interim Ecological Soil Screening Level Documents. Washington, DC: EPA, [2025]. Disponível em: <https://www.epa.gov/chemical-research/interim-ecological-soil-screening-level-documents>. Acesso em: 10 abr. 2026.

(15) - BRASIL. SÃO PAULO. [CETESB] Companhia Ambiental do Estado de São Paulo. Decisão de Diretoria N.º 125/2021/E, de 09 de dezembro de 2021. Disponível em: < https://licitacao.capaobonito.sp.gov.br/wp-content/uploads/2024/09/DD-125-2021.pdf >. Acesso em: 27 abr. 2026.

(16) - HOLANDA. National Institute for Public Health and the Environment. Ministry of Health, Welfare and Sport. Update of ecological risk limits of nickel in soil. RIVM Letter report 2015-0137 (2015). Disponível em: < https://www.rivm.nl/bibliotheek/rapporten/2015-0137.pdf >. Acesso em: 10 abr. 2026.

(17) - HOLANDA. National Institute for Public Health and the Environment. Ministry of Health, Welfare and Sport. Environmental risk limits for polycyclic aromatic hydrocarbons (PAHs): For direct aquatic, benthic, and terrestrial toxicity - RIVM report 607711007/2012 (2012). Disponível em: <https://www.rivm.nl/bibliotheek/rapporten/607711007.pdf>. Acesso em: 10 abr. 2026.

(18) - HOLANDA. National Institute for Public Health and the Environment. Ministry of Health, Welfare and Sport. Environmental risk limits for various chlorobenzenes - RIVM report 201782020/2010 (2010). Disponível em: <https://www.rivm.nl/bibliotheek/rapporten/601782020.pdf>. Acesso em: 10 abr. 2026.

(19) - HOLANDA. National Institute for Public Health and the Environment. Ministry of Health, Welfare and Sport. Ecotoxicologically based environmental risk limits for several volatile aliphatic hydrocarbons - RIVM report 601782002/2007 (2007). Disponível em: <https://www.rivm.nl/bibliotheek/rapporten/601782002.pdf>. Acesso em: 10 abr. 2026.

(20) - HOLANDA. National Institute for Public Health and the Environment. Ministry of Health, Welfare and Sport. Environmental risk limits for twelve volatile aliphatic hydrocarbons: An update considering human-toxicological data. RIVM report 601782013/2009 (2009). Disponível em: < https://www.rivm.nl/bibliotheek/rapporten/601782013.pdf >. Acesso em: 10 abr. 2026.

(21) - HOLANDA. National Institute for Public Health and the Environment. Ministry of Health, Welfare and Sport. Evaluation of ecological risk limits for DDT and drins in soil: Assessment of the direct toxicity and food chain transfer - RIVM Letter report 2015-0139 (2015). Disponível em: <https://rivm.openrepository.com/server/api/core/bitstreams/2602c856-9fbb-4138-bd73-d82b356ddd65/content>. Acesso em: 10 abr. 2026.

(22) - HOLANDA. National Institute for Public Health and the Environment. Ministry of Health, Welfare and Sport. Ecotoxicological risk limits for PFOS in soil and groundwater. Bilthoven: RIVM, 2020. (RIVM Report 2020-0085). Disponível em: <  https://www.rivm.nl/bibliotheek/rapporten/2020-0085.pdf>. Acesso em: 10 abr. 2026.

(23) - HOLANDA. National Institute for Public Health and the Environment. Ministry of Health, Welfare and Sport. Environmental risk limits for twelve substances, prioritised on the basis of indicative risk limits.Bilthoven: RIVM, 2007. (RIVM Report 601782003). Disponível em: <https://www.rivm.nl/bibliotheek/rapporten/601782003.pdf>. Acesso em: 10 abr. 2026.

(24) – Derivação feita pela Companhia Ambiental do Estado de São Paulo (CETESB).

Observação: As substâncias e seus respectivos valores orientadores apresentados neste Anexo não constituem lista exaustiva, podendo ser complementados ou atualizados conforme novas substâncias químicas sejam identificadas ou conforme evolua o conhecimento técnico-científico aplicável.